# VISVESVARAYA TECHNOLOGICAL UNIVERSITY

Jnana Sanghama, Belagavi-590018

CONTROL SYSTEMS (BEC403) PROJECT “SMART VEHICLE (SMV) ”

Submitted by,

USN 1BI22ET054

1BI22ET042

1BI22ET05

1BI22ET034

NAME SYED MAAZ SAMITH R

VINEET KRISHNA POORNESH H S

Under the guidance of Dr. Girish Kumar N G Designation

Department of ETE, BIT Carried out at

## DEPARTMENT OF ELECTRONICS AND TELECOMMUNICATION ENGINEERING BANGALORE INSTITUTE OF TECHNOLOGY

### K. R. Road, V. V. Pura, Bangalore - 560004

CONTENTS

Abstract

- Introduction

- Objective of the project

- Hardware and Software requirements

- Methodology of the Work

- Explanation of the Project Work

- Domain

- Obtained results

- Conclusion and outcomes

- Future Scope

- References

- CO-PO-PSO Mapping

Abstract

SMART VEHICLE FOR RESCUE OPERATIONS

mechanisms. Traditional rescue operations often

This project is an innovative module designed to save lives in disaster scenarios such as earthquakes, landslides, war zones, and nuclear events. The vehicle is carefully driven towards the victims to provides life-saving support, including oxygen supply and medication delivery. The module's mobility, enabled by solenoid wheels, ensures efficient navigation through challenging terrains. By offering immediate assistance and stabilizing victims' conditions, the module supports rescue teams and enhances the efficiency of disaster response operations. once reaching near the victim the sensors in the vehicle sends information such as carbon dioxide level, temperature and humidity. The information is then stored for analyzing and deciding the treatments for the victim. This technology significantly improves survival rates and health outcomes in emergencies. the SMV-Environment Sensing project represents a significant advancement in disaster response technology. By combining environmental sensing with immediate medical intervention and mobility capabilities, this module has the potential to save countless lives in critical situations. Its deployment in disaster zones can facilitate timely assistance and provide essential support to those trapped, ultimately improving survival rates and health outcomes in emergencies.

## INTRODUCTION

Natural disasters, such as earthquakes, landslides, and storms, continue to cause significant loss of life and property worldwide. The 2023 earthquake in Turkey alone claimed over 50,000 lives, illustrating the devastating impact of such events [i]. Despite the small percentage of global deaths caused by disasters—approximately 0.2%—the tragic nature of these incidents highlights the urgent need for improved rescue and support

face challenges in swiftly locating and assisting trapped individuals, leading to preventable deaths and severe injuries, particularly from respiratory complications due to limited oxygen supply.

This paper introduces the Smart Vehicle project, an innovative module designed to enhance disaster response by providing immediate and targeted support to individuals trapped under debris. The module utilizes advanced sensors to detect human presence and assess environmental conditions, enabling dynamic oxygen supply and medical intervention. By addressing critical needs such as oxygenation and respiratory support, the Smart Vehicle aims to stabilize victims' conditions until professional rescue teams can reach them.

The module's functionality extends beyond detection and oxygen delivery. It is equipped with mobility features that allow it to navigate through rubble and challenging terrains, ensuring timely assistance to those in need. This capability significantly enhances the efficiency of rescue operations, reducing the burden on rescue teams and improving overall survival rates.

This paper explores the design, functionality, and potential impact of the Smart Vehicle in disaster response scenarios. By leveraging advanced sensing technologies and immediate medical intervention, this project represents a critical advancement in the efforts to save lives and reduce the severity of injuries in disaster-affected areas.

## OBJECTIVES

- Gathering Environmental near the victim Information: Advanced sensors, including carbon dioxide (CO2), temperature, and humidity sensors, are

utilized to accurately assess the environment in which the trapped individuals are located. This critical data on the surrounding conditions will inform rescue operations and medical interventions.

- Provide Immediate Support: Based on the gathered environmental data, life- saving actions are initiated, including the dynamic supply of oxygen through a network of pipes. The condition of trapped individuals is stabilized by continuously monitoring CO2 levels and adjusting oxygen supply accordingly until professional medical assistance arrives.

- Enhance Rescue Operations: the project aims enhances rescue operations by reducing the burden on rescue teams through technology that facilitates faster and more efficient response efforts. This enables rescue teams to focus on larger- scale recovery operations while the Smart Vehicle manages immediate life support for trapped individuals. The efficiency of rescue operations is increased by providing accurate data on the location and condition of trapped individuals.

- Deliver Medical Assistance: Aims to deliver medical assistance by detecting injuries and respiratory issues in trapped individuals using the module's sensors. Necessary medical support, such as nebulization for respiratory relief, is provided to stabilize individuals experiencing breathing difficulties. Continuous monitoring and intervention are ensured to address any emerging health issues until rescue teams arrive.

- Improve Survival Rates: the project aims to improve survival rates. The condition of trapped individuals is maintained to prevent deterioration while awaiting rescue by providing

adequate oxygen supply and immediate medical support. The chances of survival and health outcomes for individuals trapped in disaster situations are enhanced through timely and effective life support. Ultimately, the number of preventable deaths and severe injuries in disaster-affected areas is reduced through the deployment of the Smart Vehicle are ensured to address any emerging health issues until rescue teams arrive.

- Moving the vehicle in the debris field: The project seeks to navigate through debris fields. The Smart Vehicle is equipped with specialized toroidal wheels, allowing it to steer effectively through challenging and uneven terrains. This mobility feature ensures that the vehicle can reach trapped individuals quickly and efficiently, even in the

most obstructed environments.

## COMPONENTS

- Electronic Components:

  - Arduino UNO: The Arduino Uno is responsible for controlling the motors that drive the specialized toroidal wheels of the Smart Vehicle. By using motor driver shields, the Arduino can manage the speed and direction of each wheel, allowing the vehicle to navigate through challenging terrains.

  - Esp32: the ESP32 microcontroller plays a crucial role in data acquisition and communication. It is connected to various environmental sensors, including carbon dioxide (CO2), temperature, and humidity sensors, and facilitates the transmission of data to the ThingSpeak platform. The integration of ESP32 enhances the project’s capabilities in real-time monitoring and remote data access.

  - Motor driver L293d: The L293D motor driver is used to control the motors that drive the specialized toroidal wheels of the Smart Vehicle. It can drive two DC motors simultaneously, allowing the vehicle to move forward, backward, and turn. The motor driver manages the power supplied to the motors, ensuring they operate smoothly and efficiently.

  - Hc-05: By integrating the HC-05 with the Arduino Uno, users can remotely control the Smart Vehicle’s movements and actions via a Bluetooth-enabled device. Commands for driving the vehicle, adjusting speed, or activating sensors can be sent from a custom app or Bluetooth terminal software on the paired device.

.

  - Mq-135: The MQ-135 is a versatile gas sensor used for detecting a range of gases, including carbon dioxide (CO2), ammonia, benzene, and others. In the Smart Vehicle project, the MQ-135 sensor is utilized to monitor CO2 levels in the environment, which is crucial for assessing the conditions around trapped individuals and providing timely support.

  - Dth-22: The DHT22 is a widely used digital sensor for measuring temperature and humidity. In the Smart Vehicle project, the DHT22 sensor plays a vital role in environmental monitoring, providing data that is crucial for assessing the conditions in which trapped individuals are situated.

  - Camera module: A camera module with Wi-Fi connectivity. This module combines a camera with wireless communication capabilities, allowing for real-time video streaming and remote surveillance.

- Electronic Components:

  - Gear motors: Gear motors are used to provide high torque at lower speeds. This

is essential for driving the Smart Vehicle through rugged and uneven terrain. The gear mechanism reduces the motor’s rotational speed while increasing the torque, allowing the vehicle to move with sufficient power to overcome obstacles.

- Hardware Components:

  - Chassis: The chassis serves as the foundational structure of the Smart Vehicle, providing a framework that supports and houses various components, including motors, sensors, and control systems. It is critical to the vehicle's functionality, stability, and overall performance.

  - Wheels: Toroidal wheels are a specialized type of wheel that feature a ring-shaped design, offering unique advantages for navigating challenging terrains. In the Smart Vehicle project, toroidal wheels are chosen for their ability to handle uneven surfaces and debris effectively.

- Software component:

  - Arduino ide: The Arduino Integrated Development Environment (IDE) is a crucial tool for programming and managing the microcontroller-based components of the Smart Vehicle project. It provides a user-friendly platform for writing, compiling, and uploading code to Arduino boards, such as the Arduino Uno and other compatible microcontrollers used in the project.

  - ThinkSpeak: ThingSpeak is a crucial component of the Smart Vehicle project, providing a robust platform for data collection, visualization, and remote monitoring. By leveraging ThingSpeak’s capabilities, the project enhances its ability to track and analyze environmental conditions, improve decision-making,

and coordinate rescue operations effectively. The platform’s real-time access, user-friendly visualization tools, and integration options significantly contribute to the success of the Smart Vehicle in disaster response scenarios.

### Circuit Diagram

To set up the circuit for wireless control of gear motors using an Arduino Uno, L293D motor driver, HC-05 Bluetooth module, and a battery, follow these steps: Connect the VCC to the 5V pin and GND to the GND pin of the Arduino. Use digital pins 3, 4, 5, and 6 of the Arduino to connect to IN1, IN2, IN3, and IN4 of the L293D motor driver, respectively. Connect pins 10 and 11 of the Arduino to EN1 and EN2 of the L293D. For serial communication, connect the RX pin of the Arduino to the TX pin of the HC-05 Bluetooth module, and the TX pin to the RX pin of the HC-05. Power the L293D motor driver by connecting VCC1 to 5V and VCC2 to the positive terminal of the battery, and ground it by connecting GND to the GND of the Arduino. Connect the motor outputs of the L293D (OUT1 and OUT2) to the terminals of the first gear motor, and OUT3 and OUT4 to the terminals of the second gear motor. The HC-05 Bluetooth module should have its VCC connected to 5V and GND to GND. Finally, connect the positive terminal of the battery to VCC2 of the L293D motor driver, and the

negative terminal to the common ground. This setup will allow the Arduino to control the gear motors via Bluetooth, with the L293D driving the motors based on the commands received.

The circuit diagram for the Smart Vehicle project involves an ESP32 microcontroller connected to a DHT22 sensor for temperature and humidity measurements and an MQ-135 sensor for CO2 detection. The DHT22's data pin connects to a GPIO pin on the ESP32, with power and ground from the ESP32's 3.3V and GND pins. Similarly, the MQ-135's analog output connects to an ADC pin on the ESP32, also powered by the ESP32's 3.3V and GND pins. This setup enables the ESP32 to gather environmental data and transmit it wirelessly to a platform like ThingSpeak for remote monitoring.

## METHODOLOGY

The methodology for the Smart Vehicle project involves a systematic approach to designing, developing, and deploying the vehicle to enhance disaster response efforts. The following steps outline the process from conception to implementation:

    - Project Planning And Requirements Gathering: Define the primary objectives of the Smart Vehicle project, including

detecting human presence, providing immediate support, enhancing rescue operations, delivering medical assistance, and improving survival rates. Gather and analyze the requirements for each component of the Smart Vehicle, including sensors, communication modules, control systems, and mobility solutions.

    - Component Selection and Design: In this stage, appropriate sensors are selected for detecting environmental conditions and monitoring health. This includes choosing the MQ-135 sensor for CO2 detection, the DHT22 sensor for temperature and humidity measurements, and camera modules with Wi-Fi connectivity. Communication modules, such as the ESP32, are selected for data transmission to ThingSpeak, facilitating remote monitoring and data visualization. The Arduino Uno is chosen for driving the vehicle and managing sensor data, while motor drivers like the L293D are integrated to control the gear motors. Specialized toroidal wheels are also incorporated to enhance the vehicle’s maneuverability through debris and challenging terrains.

    - System Integration: The integration phase involves assembling the Smart Vehicle by combining sensors, communication modules, control systems, and wheels onto the chassis. Proper mounting and alignment of all components are ensured. Code is written and uploaded to the Arduino Uno using the Arduino IDE to manage sensor data acquisition, motor control, and communication with ThingSpeak. The ESP32 is programmed to transmit sensor data to ThingSpeak, enabling real-time monitoring and visualization. The platform is configured for data visualization and alert settings.

    - Testing and Calibration: Individual components are tested to confirm they operate correctly. The accuracy of sensor

measurements and the effectiveness of communication modules are verified. Integrated system tests are conducted to ensure seamless operation of all components. The vehicle’s mobility, data transmission, and sensor performance are evaluated in simulated environments. Calibration of sensors is performed to ensure accurate readings, and thresholds and parameters for data collection and alerts are adjusted based on testing results.

    - Deployment and Operation: The Smart Vehicle is deployed in disaster zones or simulated environments to assess its performance under real conditions. The vehicle’s operation is monitored, and feedback from rescue teams is collected. Real-time data from the vehicle is analyzed using ThingSpeak, with visualizations and alerts providing insights into the vehicle’s performance. Support for the Smart Vehicle’s operation, including maintenance, troubleshooting, and updates, is provided to ensure its continued functionality and effectiveness in disaster response scenarios.

## EXPLANATION OF THE

PROJECT

The Smart Vehicle project aims to enhance disaster response capabilities by developing an autonomous vehicle equipped with advanced sensing and communication technologies. The primary goal is to provide timely support and improve survival rates for individuals trapped under debris in disaster scenarios such as earthquakes, landslides, war zones, and nuclear calamities.

### The vehicle is designed to perform the following key functions:

  - Environmental Sensing: The vehicle utilizes a combination of sensors to monitor the environmental conditions

of the area where individuals may be trapped. The DHT22 sensor measures temperature and humidity levels, while the MQ-135 sensor detects CO2 concentrations. These sensors are connected to an ESP32 microcontroller, which serves as the central processing unit for data acquisition and transmission.

  - Data Transmission and Monitoring: The ESP32 is programmed to collect data from the DHT22 and MQ-135 sensors and transmit this information to a remote server using Wi-Fi. The data is sent to the ThingSpeak platform, where it can be monitored in real-time by rescue teams. This allows for continuous	assessment	of environmental conditions, aiding in the identification of trapped individuals and the prioritization of rescue efforts.

  - Mobility and Navigation: The vehicle is equipped with specialized toroidal wheels, which provide enhanced maneuverability over debris and rough terrains commonly found in disaster zones. The Arduino Uno microcontroller is responsible for driving the vehicle, using motor drivers like the L293D to control the gear motors. This setup ensures that the vehicle can navigate complex environments to reach affected individuals efficiently.

  - Immediate Support and Medical Assistance: Upon detecting human presence, the vehicle can initiate life- saving actions such as providing oxygen and administering medical support. The vehicle is designed to supply oxygen dynamically based on the detected CO2 levels, helping to maintain respiratory health until professional medical assistance arrives. Additionally, the vehicle can

deliver nebulized medication for individuals experiencing respiratory distress.

  - Data Visualization and Alerts: The data collected by the sensors is visualized on the ThingSpeak platform, allowing for real-time monitoring and analysis. Alerts can be configured based on specific thresholds, enabling quick responses to critical situations.

### The Smart Vehicle project employs the following components and tools:

  - Sensors: DHT22 for temperature and humidity, MQ-135 for CO2 detection, and a camera module with Wi-Fi connectivity for visual monitoring.

  - ​Microcontrollers: ESP32 for sensor data processing and communication, and Arduino Uno for vehicle control.

  - Communication Modules: ESP8266 for Wi-Fi communication and data transmission to ThingSpeak.

  - Motor Drivers: L293D for controlling gear motors.

  - ​Mobility Solutions: Specialized toroidal wheels for enhanced maneuverability.

  - Software: Arduino IDE for programming the microcontrollers, and ThingSpeak for data visualization and alerts.

## DOMAIN OF THE PROJECT

The Smart Vehicle project falls within the domain of Disaster Response Technology. This interdisciplinary field combines elements from various areas, including:

- Robotics and Automation: The project involves the design and implementation of an autonomous vehicle capable of navigating complex environments and

performing tasks without direct human control. This includes the integration of specialized toroidal wheels for enhanced manoeuvrability and the use of motor drivers for precise control of the vehicle's movements.

- Environmental Sensing: The project utilizes advanced sensors to monitor environmental conditions, such as temperature, humidity, and CO2 levels. These sensors provide critical data that informs rescue operations and helps identify the presence and condition of trapped individuals.

- IoT (Internet of Things): The ESP32 microcontroller, equipped with Wi-Fi capabilities, connects the vehicle to the internet, allowing for real-time data transmission and remote monitoring. This aspect of the project leverages IoT technologies to enhance situational awareness and decision-making in disaster response.

- Wireless Communication: The project employs wireless communication modules, such as the ESP8266, to transmit sensor data to remote platforms like ThingSpeak. This enables continuous monitoring and analysis of environmental conditions from a distance.

- Embedded Systems: The integration of various sensors, microcontrollers (ESP32 and Arduino Uno), and communication modules represents a complex embedded system designed to perform specific tasks reliably and efficiently in disaster scenarios.

- Data Visualization and Analytics: By using platforms like ThingSpeak, the project includes the ability to visualize sensor data in real-time, set alerts, and analyze trends. This aspect is crucial for making informed decisions during rescue operations.

- Healthcare Technology: The project addresses the immediate medical needs of individuals trapped in disaster zones by providing oxygen and administering nebulized medication. This intersection with healthcare technology is essential for stabilizing the condition of affected individuals until professional medical help arrives.

## OBTAINED RESULTS

With the integration of our innovative idea, software, and hardware components, the Smart Vehicle project successfully accomplished many, if not all, of its objectives. Here are the key outcomes:

- Enhanced Mobility: The use of specialized toroidal wheels allowed the Smart Vehicle to navigate terrains inaccessible to normal wheels, ensuring it could reach areas trapped under debris effectively.

- Effective Environmental Monitoring: The MQ-135 sensor successfully wrote data to the ESP8266, and the ESP32 transmitted this data to the cloud platform ThingSpeak for real-time monitoring of CO2 levels.

- Seamless Data Transmission: The ESP32 effectively transmitted environmental data to ThingSpeak, enabling continuous remote monitoring and analysis.

- Precise Vehicle Control: The Arduino microcontroller, integrated with other components, allowed for precise steering of the vehicle to desired locations using an RC controller.

- Location Tracking: The vehicle was equipped with a tracker that shared its live location directly with users. An application was used to access this live location, providing real-time tracking and monitoring.

- Live Visual Feedback: The camera module provided live footage directly to a monitor or smartphone, enhancing situational awareness for rescue teams.

- Illumination: Light-emitting diodes (LEDs) successfully provided illumination, aiding in visibility during rescue operations.

- Oxygen and Water Supply: Oxygen and water were successfully transmitted to the desired locations via designated channels, providing essential life support to trapped individuals.

- Medical Support: Medicines such as Clearneb Pulmules, which enhance overall respiration, and Hyperneb Respules were successfully delivered to the desired locations, providing critical medical assistance.

These outcomes collectively contribute to the primary goal of the project: increasing the survival probability of individuals trapped in disaster scenarios. By providing immediate support, real-time monitoring, and essential medical assistance, the Smart Vehicle project enhances the effectiveness of rescue operations and improves the chances of survival for those in need.

## CONCLUSION

The Smart Vehicle project represents a significant advancement in disaster response technology, leveraging cutting-edge sensors, robust communication systems, and innovative mobility solutions to enhance the efficiency and effectiveness of rescue operations. By integrating environmental sensing, real-time data transmission, precise vehicle control, and essential medical support, the project has demonstrated its potential to significantly improve survival rates in disaster scenarios.

The successful deployment and operation of the Smart Vehicle in simulated environments underscore its practical applicability and effectiveness. The ability to monitor environmental conditions, navigate challenging terrains, and provide immediate life support underscores the comprehensive approach of the project. The outcomes achieved validate the project's objectives and highlight its potential impact on saving lives during emergencies.

Moving forward, continued refinement and testing of the Smart Vehicle will further enhance its capabilities and reliability. By incorporating feedback from field deployments and rescue teams, we can make iterative improvements to ensure the vehicle remains at the forefront of disaster response technology. Ultimately, the Smart Vehicle project aims to be an indispensable tool in disaster scenarios, providing timely support and increasing the chances of survival for those in critical need.

## FUTURE SCOPE

The Smart Vehicle project, with its current success in achieving primary objectives, presents numerous opportunities for further development and enhancement. Future scopes include:

  - Integration of Advanced Drilling Mechanisms**: Adding a drill to the vehicle will enable it to penetrate through tougher debris, allowing for more effective access to trapped individuals. This will significantly enhance the vehicle’s capability to reach victims who are otherwise inaccessible.

  - Enhanced AI and Machine Learning**: Implementing advanced AI and machine learning algorithms can improve the vehicle's autonomous navigation,	decision-making capabilities, and adaptability todifferent disaster scenarios. This will allow the vehicle to learn from its environment and optimize its performance in real-time.

  - Improved Sensor Array: Expanding the array of sensors to include gas detectors for other hazardous gases, seismic activity sensors, and more precise biometric sensors can provide a more comprehensive understanding of the environment and the condition of trapped individuals.

  - Extended	Communication Capabilities:		Enhancing communication modules to support multiple network types (e.g., 5G, satellite communication) can ensure uninterrupted data transmission even in the most challenging conditions. This will enable continuous monitoring and faster response times.

  - Enhanced Battery Life and Power Management: Developing more

efficient power management systems and incorporating high-capacity batteries or renewable energy sources (e.g., solar panels) can extend the operational time of the vehicle, allowing it to function longer in disaster zones.

  - Robust Design for Harsh Environments: Further ruggedizing the vehicle to withstand extreme temperatures, water, and physical shocks will ensure its reliability and durability in various disaster scenarios.

  - User-Friendly Interfaces: Developing more intuitive and user-friendly control and monitoring interfaces, including mobile applications and VR/AR support, will enable rescue teams to operate the vehicle more effectively and gain better situational awareness.

  - Collaborative Swarm Technology: Deploying multiple vehicles that canwork collaboratively as a swarm can cover larger areas more efficiently and perform coordinated tasks, such as simultaneous debris removal and victim monitoring.

  - Real-World Testing and Deployment: Conducting extensive field tests in various simulated and real-world disaster environments will help refine the vehicle’s performance and reliability. Partnerships with disaster response organizations for pilot deployments will provide valuable feedback and pave the way for large- scale adoption.

  - Scalability and Cost Reduction: Focusing on scalable manufacturing techniques and cost-effective materials can make the Smart Vehicle more accessible to a wider range of disaster response teams and organizations globally.

By pursuing these future scopes, the Smart Vehicle project can continue to evolve and become an even more powerful tool in saving lives during disaster situations.

## REFERENCE

### i.	Worlddata.info

- CO – PO – PSO Mapping

# Course Outcomes :

  - COI: Understand the basics of control systems to design and simulate mathematical models by using block diagram reduction and signal flow graphs techniques to determine the transfer functions.

  - CO2: Analyze the time response of first and second-order systems, Simulate and interpret time response specifications.

  - CO3: Assess the stability of a system using Routh stability criterion, root locus techniques, Bode plots, and Nyquist plots through simulation results

  - CO4: Apply the concepts of state, state variables, and state models to represent and solve state equations for linear continuous-time systems.

  - CO5: Design and develop an open loop or closed loop control system, applying knowledge on control systems through project-based assignments, fostering practical application and problem- solving skills.

# Program Outcomes :

- ENGINEERING KNOWLEDGE: Apply the knowledge of Mathematics, Science, Engineering Fundamentals, and an Engineering specialization to the solution of complex engineering problems.

- PROBLEM ANALYSIS: Identify, formulate, research literature, and analyze complex engineering problems reaching substantiated conclusions using first principles of mathematics, natural sciences, and engineering sciences.

- DESIGN/DEVELOPMENT OF SOLUTION: Design solutions for complex engineering problems and design system components or processes that meet the specified needs with appropriate consideration for the public health and safety, and the cultural, societal, and environmental considerations.

- CONDUCT INVESTIGATIONS OF COMPLEX PROBLEM: Use research-based knowledge and research methods including design of experiments, analysis and interpretation of data, and synthesis of the information to provide valid conclusions.

- MODERN TOOL USAGE: Create, select, and apply appropriate techniques, resources, and modern engineering and IT tools including prediction and modelling to complex engineering activities with an understanding of the limitations.

- THE ENGINEER AND SOCIETY: Apply reasoning informed by the contextual knowledge to assess societal, health, safety, legal and cultural issues and the consequent responsibilities relevant to the professional engineering practice.

- ENVIRONMENT AND SUSTAINABILITY: Understand the impact of the professional engineering solutions in societal and environmental contexts, and demonstrate the knowledge of, and need for sustainable development.

- ETHICS: Apply ethical principles and commit to professional ethics and responsibilities and norms of the engineering practice.

- INDIVIDUAL AND TEAM WORK: Function effectively as an individual, and as a member or leader in diverse teams, and in multidisciplinary settings.

- COMMUNICATION: Communicate effectively on complex engineering activities with the engineering community and with society at large, such as, being able to comprehend and write effective reports and design documentation, make effective presentations, and give and receive clear instructions.

- PROJECT MANAGEMENT AND FINANCE: Demonstrate knowledge and understanding of the engineering and management principles and apply these to one's own work, as a member and leader in a team, to manage projects and in multidisciplinary environments.

- LIFE-LONG LEARNING: Recognize the need for, and have the preparation and ability to engage in independent and life-long learning in the broadest context of technological change.

# Program Specific Outcomes :

  - PSO 1: Analyse and Design Analog and Digital modules for a given specification and function.

  - PSO 2: Implement functional blocks of Hardware-Software co-designs for Embedded Systems, Signal Processing, Communication and Networking applications.

## CO-PO-PSO MAPPING

|  | POs |  |  |  |  |  |  |  |  |  |  |  | PSOs |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl.No | PO1 | PO2 | PO3 | PO4 | PO5 | PO6 | PO7 | PO8 | PO9 | PO10 | PO11 | PO12 | PSO1 | PSO2 |
| CO1 | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| CO2 | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| CO3 | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| CO4 | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
| CO5 | 2 | 1 | 2 | 2 | 3 | 3 | 2 | 3 | 3 | 2 | 3 | 3 | 3 | 2 |

Mapping Scale

- Low

- Medium 3-High